252 Straipsnis - 2010 Bendrieji reikalavimai serijiniams automobiliams (Grupė N), turistiniams automobiliams (Grupė A), didžiojo turizmo automobiliams (Grupė B) General Prescriptions for Production Cars (Group N), Touring Cars (Group A), Grand Touring Cars (Group B)

| 1 Straipsnis: Bendrosios pastabos 1.1 Modifikacijos Draudžiamos visos modifikacijos, išskyrus atvejus, kai tą atskirai leidžia grupės, kurioms priskiriamas automobilis, taisyklės arba žemiau pateikti bendrieji reikalavimai arba skyriuje “Saugos įranga” numatyti reikalavimai. Automobilio konstrukciniai komponentai turi išlaikyti savo originalias funkcijas. 1.2 Bendrųjų reikalavimų taikymas Bendrųjų reikalavimų reikia laikytis tokiose serijinių automobilių (Grupė N), turistinių automobilių (Grupė A), didžiojo turizmo automobilių (Grupė B) lenktynėse, kuriose nekeliami griežtesni reikalavimai. 1.3 Medžiagos Draudžiama naudoti medžiagas, kurių konkretus našumo koeficientas yra didesnis negu 40 Gpa/g/cm3, išskyrus žvakes, išmetimo sistemos apsaugą nuo perkaitimo, vandens siurblio turbinos jungtis, stabdžių kaladėles, riedančius guolių elementus (rutulius, adatas, ritinius), elektronikos elementus ir daviklius, detales, sveriančias mažiau kaip 20 g, ir padengimus, kurių storis yra mažesnis kaip 10 mikronų. Visų detalių, kurioms apibojimai netaikomi arba kurios yra aprobuotos pasirenkamajame variante (OV) gamybai draudžiama naudoti metalines medžiagas, kurių takumo koeficientas yra didesnis kaip 30 Gpa/g/cm3 ar kurių specifinis UTS yra didesnis kaip 0,24 Mpa/kg/m3 neturinčioms sudėtyje geležies ir 0,3 Mpa/kg/m3 geležinėms medžiagoms (apie 80% plieno). Leidžiama naudoti Ti-6Al-4V ASTM grade 5 tipo titano lydinį (5,5<Al<6,75, C maks. 0,10, 3,5<V<4,5, 87,6<ti<91), išskyrus tas atskiras detales, kurioms titaną naudoti griežtai draudžiama. Nesisukančios turbinos ar kitų ekvivalentiškų turbo pripūtimo sistemų dalys (išskyrus riedančias guolių detales) gali būti pagamintos iš keraminės medžiagos arba turėti keraminį padengimą. Šie apribojimai netaikomi detalėms, homologuotoms kartu su standartiniu automobiliu. Draudžiama naudoti magnio lydinių lakštus, kurių storis mažesnis nei 3 mm. 1.4 Kiekvieno varžybų dalyvio pareiga – įrodyti lenktynių inspektoriams ir teisėjams, jog jo automobilis pilnai atitinka šių taisyklių reikalavimus ir atitiks bet kuriuo lenktynių momentu. 1.5 Sugadintas sriegis gali būti taisomas iš naujo įsriegiant naują sriegį su tokiu pačiu vidiniu skersmeniu (“helikoidinio” tipo). 1.6 Visi A grupės automobiliai, homologuoti po 99.01.01, išskyrus komplektinius variantus, bei lenktyniaujantys raliuose turi būti ne platesni kaip 1800 mm. N grupės automobiliai gali lenktyniauti jų įdiegtose versijose. 1.7 „Laisvos detalės“ Laisva detalė suprantama taip, kad originali detalė gali būti pakeista nauja detale su salyga, kad naujoji detalė neturės papildomų funkcijų, kurių neatlieka originali detalė. 2 straipsnis: Matmenys ir masė 2.1 Prošvaisa Nė viena automobilio dalis neturi liesti žemės, kai visos vienos pusės padangos visiškai be oro. Šis patikrinimas atliekamas lygioje vietoje lenktynių sąlygomis (su žmonėmis). 2.2 Balastas Automobilio masę leidžiama papildyti vienu ar keletu balastų, su sąlyga, kad tai yra patvarūs monolitiniai blokai, pritvirtinti tvirtinimo detalėmis, netrukdančiomis užplombuoti. Balastas privalo būti pritvirtintas prie kabinos grindų, matomas ir lenktynių teisėjų užplombuotas. Pritaikymas: Turistiniai automobiliai (Grupė A), didžiojo turizmo automobiliai (Grupė B) ir R grupės automobiliai. Serijinės gamybos automobiliams (Grupė N) neleistinas jokios rūšies balastas. Tačiau ralio varžybose automobilio kabinoje ir / arba variklio skyriuje ir / arba bagažinėje gali būti įrankiai ir automobilio atsarginės dalys, laikantis 253 straipsnio sąlygų. 3 Straipsnis: Variklis 3.1 Turbo pripūtimas Turbo pripūtimo atveju nominalus cilindrų tūris bus dauginamas iš 1,7 koeficiento benzininiams varikliams ir 1,5 – dyzeliniams varikliams, ir apskaičiavus tariamą tūrį, automobilis pateks į atitinkančią klasę. Automobilis bus vertinamas taip, tartum tokiu būdu padidintas cilindrų tūris būtų realus jų tūris. Priskiriant automobilį jo cilindrų tūrio klasei bus atskirai vertinami jo vidiniai matmenys, vietų skaičius, minimali masė ir t.t. 3.2 Variklių su slenkančio judesio stūmokliais ir rotorinių variklių ekvivalentiškumo formulė (iš NSU Wankel’io patentuotų variklių tipo). Ekvivalentinis tūris yra lygus tūriui, apibrėžiamam kaip skirtumui tarp maksimalaus ir minimalaus degimo kameros tūrio. 3.3 Variklių su slenkančio judesio stūmokliais ir turbininių variklių ekvivalentiškumo formulė Formulė tokia: S(3.10 x R) - 7.63 C = ¾¾¾¾¾¾¾¾¾ 0.09625 S = aukšto slėgio purkštuko plotas (išreikštas kvadratiniais centimetrais), kuriuo apibūdinamas oro srauto, išeinančio nuo statoriaus menčių (arba iš pirmosios pakopos, jei statorius turi keletą pakopų) plotas. Matavimai atliekami imant plotą tarp aukšto slėgio turbinos pirmosios pakopos menčių. Tuo atveju, kai pirmosios pakopos statoriaus mentės yra reguliuojamos, jos turi būti atidarytos iki didžiausių matmenų. Taigi, aukšto slėgio purkštuko plotas yra lygus aukščio (išreikšto centimetrais), pločio (išreikšto centimetrais) ir menčių skaičiaus sandaugai. R = Slėgio santykis yra turbininio variklio kompresoriaus santykis. Jis gaunamas kartu dauginant kiekvienos kompresoriaus pakopos reikšmes, kaip tai paaiškinta žemiau: Ikigarsinis ašinis kompresorius: 1,15 pakopai Viršgarsinis ašinis kompresorius 1,5 pakopai Radialinis kompresorius 4,25 pakopai. Taigi, kompresorius, turintis vieną radialinę ir šešias ašines ikigarsines pakopas bus apibrėžiamas slėgio santykiu: 4,251,151,151,151,151,151,15 arba 4,25 (1,15)6. C = Ekvivalentinis variklio su slenkančiojo judesio stūmokliais tūris, cm³. 3.4 Draudžiami visi varikliai, kuriuose kuras įpurškiamas ir dega pasroviui per išmetimo sistemos angą. 3.5 Variklių su slenkančio judesio stūmokliais ir naujo tipo variklių ekvivalentiškumas FIA pasilieka sau teisę daryti pakeitimus remiantis palyginimais, tarp klasikinio variklio ir naujo tipo variklių, su dviejų metų įspėjamuoju laikotarpiu nuo sausio mėnesio 1 dienos tų metų, kada buvo priimtas toks sprendimas. 3.6 Išmetimo sistema ir duslintuvas Net ir tuo atveju, kai specifiniai reikalavimai tam tikrai grupei leidžia pakeisti originalų duslintuvą, automobiliai, dalyvaujantys lenktynėse viešaisiais keliais, visuomet privalo būti su išmetimo sistemos duslintuvu, atitinkančiu šalies, kurioje vyksta lenktynės, kelių eismo taisykles. Visų raliuose dalyvaujančių automobilių keliamo triukšmo lygis atvirame kelyje neturi viršyti 103 dB(A), kai variklio sukimosi greitis yra 3500 aps/min benzininiam varikliui ir 2500 aps/min dyzeliniam varikliui , išskyrus tuos atvejus, kai vietinėse taisyklėse yra nurodytos žemesnės triukšmo ribos. Išmetimo vamzdžio anga turi būti ne aukščiau nei 45 cm ir ne žemiau nei 10 cm nuo žemės. Išmetimo vamzdžio anga turi išsitekti automobilio perimetre arba ne daugiau nei 10 cm už šio perimetro ir turi būti už vertikalios plokštumos, einančios per ratų bazės centrą. Be to, turi būti užtikrinta reikiama apsauga, sauganti nuo apsideginimo prisilietus prie įkaitusių vamzdžių. Išmetimo sistema neturi būti laikina. Išmetamosios dujos gali išeiti tik per išmetimo sistemos galą. Važiuoklės dalys neturi būti naudojamos išmetamųjų dujų šalinimui. Katalizinis išmetimas: Vienam automobilio modeliui turėtų būti patvirtinti du galimi variantai (katalizuojamas ir kitoks išmetimas), automobilis privalo atitikti vienos arba kitos versijos keliamus reikalavimu, draudžiama bet kokia dviejų variantų kombinacija. Visi komplektinio varianto (VK- WRC- S2000- Rally) automobiliai privalo turėti sumontuotą homologuotą katalizinį išmetimą. Visų grupių automobiliai privalo turėti sumontuotą originalų arba homologuotą katalizinį išmetimą, jei jie yra registruoti šalyse, kur tai privaloma. Jei tai nėra privaloma varžybas organizuojančioje šalyje, katalizinį išmetimas gali būti nuimtas. Kokios nors homologuoto katalizinio konverterio modifikacijos yra draudžiamos Kontrolieriams privalo buti pateikta katalizatoriaus sertifikato kopija. 3.7 Automobilio užvedimas Tai atliekama starteriu su elektriniu ar kitokiu automobilyje esančiu energijos šaltiniu, valdomu savo vietoje sėdinčio vairuotojo. 3.8 Cilindrai Leidžiama remontuoti beįvorinių variklių cilindrus pridedant medžiagą, bet ne detales. 4 Straipsnis: Transmisija Visi automobiliai privalo turėti pavarų dėžę, įskaitant atbulinę pavarą, kuri turi būti veikianti lenktynių pradžioje, ir kurią vairuotojas galėtų valdyti normaliai sėdėdamas savo vietoje. 5 Straipsnis: Pakaba Pakabos dalys, dalinai arba pilnai pagamintos iš sudėtinių medžiagų, yra draudžiamos. 6 Straipsnis: Ratai Ratai, dalinai arba pilnai pagaminti iš sudėtinių medžiagų yra draudžiami. Ratų pločio matavimas: Turi būti matuojamas automobilyje sumontuoto, į žemę besiremiančio rato plotis, automobilis lenktynių būklėje, su vairuotoju; matuojama bet kurioje padangos perimetro vietoje, išskyrus vietą, kuria padanga remiasi į žemę. Kai sudvigubintos padangos montuojamos kaip sukomplektuoto rato dalis, jos turi atitikti maksimalius toje grupei, kurioje tos padangos naudojamos, keliamus reikalavimus (žr. 255-5.4 ir 256-5 straipsnius). 7 Straipsnis: Kėbulas / važiuoklė / Kėbulo rėmas 7.1 Automobiliai su konvertuojamais kėbulais visais atžvilgiais turi atitikti reikalavimus, keliamus atviriems automobiliams. Be to, automobiliai su standžios konstrukcijos sulankstomu stogu, varžybose privalo būti naudojami tik su uždarytu ir užrakintu stogu 7.2 Minimalūs vidiniai matmenys Jeigu J priedo leistos modifikacijos turi įtakos matmenims, nurodytiems aprobavimo formoje, tokie matmenys nelaikomi automobilio tinkamumo įvertinimo kriterijais. 7.3 Kabina Vairavimo vietos pakeitimas yra galimas, su ta sąlyga, kad originalus ir modifikuotas automobilis yra mechaniškai ekvivalentiški ir naudojamos dalys gaunamos iš gamintojo, kad galima būtų atlikti tokią modifikaciją Šiuo atveju vairo kolonėlė privalo pereiti pro kėbulą tik toje vietoje, kur yra numatyta gamintojo gaminamuose modeliuose. Super 1600, Super 2000 ir WRC klasių automobiliuose vairavimo vietos pakeitimas gali būti pasiektas tik naudojant pilnos vairavimo sistemos gamintojo pasirenkamajame variante (VO) homologuotą gaminį. Skylė automobilio kėbule, skirta pakeistai vairo kolonėlei, privalo būti homologuota kartu su šia sistema. Kabinoje gali būti įrengta tik tokia įranga: atsarginiai ratai, įrankiai, atsarginės dalys, saugos įranga, ryšio priemonės, balastas (jei jis leidžiamas), stiklų plovimo įtaiso vandens bakelis (Tik turistiniuose (Grupė A) ir didžiojo turizmo (Grupė B) automobiliuose). Visos atsarginės dalys ir įrankiai turi būti pritvirtinti už vairuotojo ir / arba šturmano sėdynių arba po jomis. Atvirų automobilių keleivių zona ir sėdynės neturi būti kaip nors uždengtos. Kabinoje esančios šalmų ir įrankių dėžės turi būti pagamintos iš nedegių, gaisro atveju neskleidžiančių toksinių garų medžiagų. Originalus oro pagalvių tvirtinimas gali būti pašalintas nekeičiant kėbulo išvaizdos. 7.4 Visi automobilio kėbulo ir važiuoklės / kėbulo rėmo paneliai turi būti iš tokios pat, kaip aprobuoto automobilio, medžiagos ir būti tokio pat storio, kaip ir originalaus aprobuoto automobilio. . Draudžiamas bet koks cheminis apdirbimas 7.5 Priekinių žibintų tvirtinimas ir apsauga Priekinėje kėbulo dalyje leidžiama gręžti kiaurymes žibintų kronšteinams, tačiau tik jų tvirtinimui. Raliuose ant žibintų gali būti uždedami šviesos neatspindintys iš lanksčios medžiagos pagaminti apsauginiai gaubtai; jie neturi išsikišti virš žibinto stiklo daugiau nei 10 cm. 7.6 - Visi pavojingi daiktai (degios medžiagos) turi būti laikomi ne kabinoje. 7.7 Purvasaugiai (tik raliuose) Galima sumontuoti skersinius purvasaugius, atitinkančius žemiau išvardintus reikalavimus. Jei purvasaugiai yra privalomi, šis reikalavimas turi būti paminėtas papildomuose varžybų nuostatuose. Visais atvejais skersiniai purvasargiai turi atitikti šiuos reikalavimus: - Jie turi būti pagaminti iš lanksčių medžiagų. - Jie turi uždengti ne mažiau nei kiekvieno rato plotį, bet mažiausiai 1/3 automobilio pločio dalis tarp priekinių ir galinių ratų turi būti neuždegta (žr. pav. 252-6). | ARTICLE 1 : GENERAL REMARKS 1.1 Modifications All modifications are forbidden unless expressly authorised by the regulations specific to the group in which the car is entered or by the general prescriptions below or imposed under the chapter "Safety Equipment". The components of the car must retain their original function. 1.2 Application of the general prescriptions The general prescriptions must be observed in the event that the specifications of Production Cars (Group N), Touring Cars (Group A), Grand Touring Cars (Group B) do not lay down a more strict prescription. 1.3 Material The use of a material which has a specific yield modulus greater than 40 Gpa/g/cm3 is forbidden, with the exception of plugs, exhaust coatings, water pump turbo joints, brake pads, brake calliper piston coatings, rolling elements of bearings (balls, needles, rollers), electronic components and sensors, parts weighing less than 20 g and all coatings with a thickness less than or equal to 10 microns. The use of a metallic material which has a specific yield modulus greater than 30 Gpa/g/cm3 or of which the maximum specific UTS is greater than 0.24 Mpa/kg/m3 for non-ferrous material and 0.30 Mpa/kg/m3 for ferrous materials (i.e. 80% iron) is forbidden for the making of all the parts that are free or homologated as an Option Variant. Ti-6Al-4V ASTM grade 5 type titanium alloy (5.5< Al <6.75, C max 0.10, 3.5 <V< 4.5, 87.6<ti<.91) is authorised, except for certain parts for which titanium is expressly forbidden. No turning part of a turbocharger or of any equivalent supercharging system (except the rolling parts of the bearings) may be made from ceramic material or have a ceramic coating. These restrictions do not concern the parts homologated with the standard vehicle. The use of magnesium alloy sheet metal with a thickness less than 3 mm is prohibited. 1.4 It is the duty of each competitor to satisfy the Scrutineers and the Stewards of the meeting that his automobile complies with these regulations in their entirety at all times during the event. 1.5 Damaged threads can be repaired by screwing on a new thread with the same interior diameter ("helicoil" type). 1.6 Any Group A car, homologated after 01.01.99, with the exception of kit variants, and competing in rallies must not be wider than 1800 mm. Group N cars may compete in their integral version. 1.7 "Free" part "Free" means that the original part, as well as its function(s), may be removed or replaced with a new part, on condition that the new part has no additional function relative to the original part. ARTICLE 2 : DIMENSIONS AND WEIGHT 2.1 Ground clearance No part of the car must touch the ground when all the tyres on one side are deflated. This test shall be carried out on a flat surface under race conditions (occupants on board). 2.2 Ballast It is permitted to complete the weight of the car by one or several ballasts provided that they are strong and unitary blocks, fixed by means of tools with the possibility to fix seals, placed on the floor of the cockpit, visible and sealed by the scrutineers. Application: Touring Cars (Group A), Grand Touring Cars (Group B) and Group R cars. No kind of ballast is authorised in Production Cars (Group N). In rallies, however, the carrying of tools and spare parts for the car in the cockpit and/or inside the engine bay and/or inside the boot only will be allowed under the conditions laid down in Article 253. ARTICLE 3 : ENGINE 3.1 Supercharging In case of supercharging, the nominal cylinder-capacity will be multiplied by 1.7 for petrol engines and by 1,5 for diesel engine, and the car will pass into the class corresponding to the fictive volume thus obtained. The car will be treated in all respects as if its cylinder-capacity thus increased were its real capacity. This shall particularly be the case for assigning the car to its cylinder-capacity class, its interior dimensions, its minimum number of places, its minimum weight, etc. 3.2 Equivalence formula between reciprocating piston and rotary engines (of the type covered by the NSU Wankel patents) The equivalent cubic capacity is equal to the volume determined by the difference between the maximum and minimum capacities of the combustion chamber. 3.3 Equivalence formula between reciprocating piston and turbine engines The formula is the following: S(3.10 x R) - 7.63 C = ¾¾¾¾¾¾¾¾¾ 0.09625 S = High pressure nozzle area - expressed in square centimetres by which is meant the area of the airflow at the exit from the stator blades (or at the exit from the first stage if the stator has several stages). Measurement is done by taking the area between the fixed blades of the high-pressure turbine first stage. In cases where the first stage turbine stator blades are adjustable, they must be opened to their greatest extent. The area of the high-pressure nozzle is thus the product of the height (expressed in cm) by the width (expressed in cm) and by the number of blades. R = The pressure ratio is the ratio of the compressor of the turbine engine. It is obtained by multiplying together the value for each stage of the compressor, as indicated hereafter: Subsonic axial compressor: 1.15 per stage Trans-sonic axial compressor: 1.5 per stage Radial compressor: 4.25 per stage. Thus a compressor with one radial and six axial subsonic stages will be designated to have a pressure ratio of: 4.25 x 1.15 x 1.15 x 1.15 x 1.15 x 1.15 x 1.15 or 4.25 x (1.15)6. C = Equivalent cubic capacity for reciprocating piston engines in cm3. 3.4 All engines into which fuel is injected and burned downstream of an exhaust port are prohibited. 3.5 Equivalencies between reciprocating piston engines and new types of engines The FIA reserves the right to make modifications on the basis of comparisons established between classic engines and new types of engines, by giving a two-year notice from the 1st January following the decision taken. 3.6 Exhaust system and silencer Even when the specific provisions for a group allow the replacement of the original silencer, the cars competing in an open-road event shall always be equipped with an exhaust silencer complying with the traffic regulations of the country(ies) through which the event is run. For all cars used in Rallies and unless the limits imposed by the local authorities are lower, the noise level on the open road must not exceed 103 dB(A) for an engine rotation speed of 3500 rpm for petrol engines and 2500 rpm for diesel engines. The orifices of the exhaust pipes shall be placed at a maximum of 45 cm and a minimum of 10 cm from the ground. The exit of the exhaust pipe must be situated within the perimeter of the car and less than 10 cm from this perimeter, and aft of the vertical plane passing through the centre of the wheelbase. Moreover, adequate protection must be provided in order to prevent heated pipes from causing burns. The exhaust system must not be provisional. Exhaust gas may only exit at the end of the system. Parts of the chassis must not be used to evacuate exhaust gasses. Catalytic exhausts: Should two possible versions of one car model be homologated (catalytic and other exhaust), the cars must comply with one or other version, any combination of the two versions being prohibited. All cars equipped with a kit (VK – WRC – S2000-Rally) must be fitted with a homologated catalytic exhaust. For all groups, all cars must be fitted with an original or homologated catalytic exhaust if this is obligatory in the country in which they are registered, unless the catalytic exhaust is not obligatory in the organising country, in which case it may be removed. No modifications to a homologated catalytic converter are allowed. An authentic copy of the homologation document must be presented to the scrutineers for the event. 3.7 Starting on board the vehicle Starter with electric or other source of energy on board operable by the driver when seated in the seat. 3.8 Cylinders For non-sleeved engines, it will be possible to repair the cylinders by adding material, but not parts. ARTICLE 4 : TRANSMISSION All cars must be fitted with a gearbox including a reverse gear which must be in working order when the car starts the event, and be able to be operated by the driver when he is normally seated. ARTICLE 5 : SUSPENSION Suspension parts made partially or entirely from composite materials are prohibited. ARTICLE 6 : WHEELS Wheels made partially or entirely from composite materials are prohibited. Measuring wheel width: The width is to be measured with the wheel mounted on the car, on the ground, the vehicle in race condition, driver aboard, at any point along the circumference of the tyre, except in the area in contact with the ground. When multiple tyres are fitted as part of a complete wheel, the latter must comply with the maximum dimensions for the Group in which these tyres are used (see Articles 255-5.4 and 256-5). ARTICLE 7 : BODYWORK / CHASSIS / BODYSHELL 7.1 Convertible vehicles must comply in all respects with the specifications applying to open cars. In addition, cars with a rigid retractable roof must be driven exclusively with the roof closed and locked up. 7.2 Minimum inside dimensions If a modification authorised by Appendix J affects a dimension stated on the homologation form this dimension may not be retained as an eligibility criterion for the car. 7.3 Cockpit Inversion of the driving side is possible, on condition that the original car and the modified car are mechanically equivalent and that the parts used are provided by the manufacturer for such a conversion for the family concerned. In particular, the steering column must pass through the bodyshell only via the hole made for that purpose by the manufacturer for the family concerned. For Super 1600, Super 2000 and WRC cars, the inversion of the driving side will be obtained by a complete steering system homologated in option variant by the manufacturer. The hole allowing the passage of the steering column through the bodyshell must be homologated with this system. Only the following accessories may be installed in the cockpit: spare wheels, tools, spare parts, safety equipment, communication equipment, ballast (if permitted), windscreen washer water container (Touring Cars (Group A) and Grand Touring Cars (Group B) only). All spare parts and tools must be fixed either behind the driver’s and/or co-driver’s seats or underneath the driver’s and/or co-driver’s seats. The passenger area and seat of an open car must in no way be covered. Containers for helmets and tools situated in the cockpit must be made of non-inflammable material and they must not, in case of fire, give off toxic vapours. The original fitting of the air bags may be removed, without modifying the appearance of the bodywork. 7.4 All bodywork and chassis / bodyshell panels of the vehicle must be at all times of the same material as those of the original homologated car and must be of the same material thickness as that of the original homologated car. All chemical treatments are forbidden. 7.5 Headlamp mounting and protection The boring of holes in the front bodywork for light brackets is authorised, limited solely to mountings. In rallies, non-reflecting protectors made from flexible material may be mounted on the headlamps ; they must not protrude forwards beyond the headlamp glass by more than 10 cm. 7.6 Any object of a dangerous nature (inflammable products, etc.) must be carried outside the cockpit. 7.7 Mud flaps (in Rallies only) It is possible to fit transversal mud flaps in conformity with the article below. If transversal mud flaps are mandatory, this requirement must be mentioned in the supplementary regulations of the event. In any case, transversal mud flaps are accepted under the following conditions: - They must be made from flexible material. - They must cover at least the width of each wheel, but at least one third of the width of the car (see Drawing 252-6) must be free behind the front wheels and the rear wheels. |
| --- | --- |
| 252-6 |  |
| - Priešais užpakalinius ratus turi būti ne mažesnis nei 20 cm tarpas tarp kairiojo ir dešiniojo purvasaragio. - Šių purvasargių apačia turi būti ne daugiau nei 10 cm nuo žemės, kai automobilis stovi be žmonių jame. - Aukščiau ir virš viso padangos aukščio, mažiausiai trys ketvirtadaliai padangos pločio privalo būti uždengta (žiūrint iš galo) - Vertikalioje projekcijoje šie purvasargiai neturi išsikišti už automobilio perimetro ribų. Purvasargiai, saugantys nuo aptaškymo iš priekio, pagaminti iš lanksčių medžiagų, gali būti sumontuoti automobilio priekyje, jei papildomos lenktynių taisyklės leidžia ar nurodo juos naudoti. Jie neturi išsikišti už automobilio kėbulo matmenų arba ne daugiau nei 10 cm į priekį nuo originalaus automobilio kėbulo matmenų, ir prieš priekinius ratus turi būti neuždengta ne mažiau nei 1/3 automobilio pločio. 8 STRAIPSNIS : Elektros sistema 8.1 Apšvietimas Priešrūkiniai žibintai gali būti pakeisti kitokiais žibintais ir atvirkščiai, su sąlyga, kad originalus jų tvirtinimas išlieka nepakitęs. 8.2 Generatoriai ir generatoriai - starteriai Generatorių ir generatorių - starterių tvirtinimui apribojimai netaikomi. 8.3 Signalas Tik raliuose signalo triukšmo lygis turi būti didesnis arba lygus 97 dB, trunkantis mažiausiai 3 sekundes ir matuojant 7 m atstumu nuo automobilio priekio. 9) staipsnis: Kuras – degalai 9.1 Kuras turi būti komercinis benzinas, pilamas iš aptarnavimo stoties siurblio, be priedų, išskyrus kasdieninėje prekyboje įsigyjamus tepalus. Kuras turi atitikti tokius reikalavimus: -Bešvinis benzinas: maksimaliai - 102,0 realus oktaninis skaičius (RON) ir 90,0 žyminis oktaninis skaičius (MON); minimaliai - 95,0 RON ir 85,0 MON -Švinuotas benzinas: maksimaliai - 100,0 RON ir 92,0MON; minimaliai - 97,0 RON ir 86,0 MON. Matavimai atliekami pagal ASTM D 2699-86 ir D 2700-86 standartus. - Tankis tarp 720 ir 785 kg/m³, 15°C temperatūroje (matuojama pagal ASTM D 4052). -Ne daugiau nei 2,8% deguonies pagal masę (arba 3,7%, jei švino kiekis yra mažesnis nei 0,013g/l) ir 0,5% azoto pagal masę. Likusią kuro dalį sudaro angliavandeniliai, jame neturi būti galingumą didinančių priedų. Azoto kiekio matavimas atliekamas pagal ASTM D 3228 standartą, deguonies kiekis nustatomas elementine analize su 0,2% paklaida. -Maksimalus peroksidų ir azoto oksidų kiekis: 100 ppm (ASTM D 3703 arba jei tuo metodu nustatyti nėra galimybės - UOP 33-82). Maksimalus švino kiekis: 0,40 g/l arba pagal šalies, kurioje vyksta lenktynės, standartą, jei jis nustato mažesnį kiekį (ASTM D 3341 arba D 3237). Maksimalus benzolo kiekis: 5% pagal tūrį (ASTM D 3606). Maksimalus garų slėgis pagal Reidą: 900 hPa (ASTM D 323)/ Distiliacija 70°C temperatūroje: 10% ÷ 47% (ASTM D 86) Distiliacija 100°C temperatūroje: 30% ÷ 70% (ASTM D 86). Distiliacija 180°C temperatūroje: ne mažiau 85% (ASTM D 86) Maksimali galutinė virimo temperatūra: 225°C (ASTM D 86) Maksimalus likutis: 2% pagal tūrį (ASTM D 86). Kuras priimamas arba atmetamas remiantis ASTM D 3244 standartu, praeinamumo riba 95%. Automobiliams su katalitiniais konverteriais draudžiamas švinuotas benzinas Jei lenktynių vietoje tiekiamo kuro kokybė neatitinka reikalavimų, organizuojančios šalies ASN privalo prašyti FIA laikino leidimo naudoti kurą, kurio charakteristikos neatitinka aukščiau nurodytų. 9.2 Dyzelinas Dyzelinių variklių kuras turi atitikti tokius reikalavimus: Angliavandenilių kiekis, % pagal masę 90,0 min. Tankis, kg/m³ 860 maks. Cetaninis skaičius (ASTM D 613) 55 maks. Skaičiuotinas cetaninis skaičius (ASTM D 976-80) 55 maks. Sieros kiekis 50 mg/kg maks. (pr-EN-ISO/DIS 14596),pagal 98/70/CE direktyvą. 9.3 Oksidantas Kaip oksidatorius su kuru gali būti maišomas tik oras. 9.4 Kuro užpildymo procedūra Standartizuotas prijungimas - Trasoje esančios centralizuotos sistemos arba varžybų dalyvių užtikrinamos sistemos atveju užpylimo žarnos turi turėti sandarius sujungimus, pritaikytus prie standartizuotų automobiliuose esančių užpylimo angų (pagal 252-5 brėžinį; vidinis skersmuo D neturi viršyti 50 mm). - Visi automobiliai privalo turėti kuro užpylimo angas, atitinkančias šį brėžinį. Sandarūs sujungimai turi atitikti savaiminio veikimo principą ir dėl to neturi turėti jokių atviroje padėtyje fiksuojamų įtaisų (spyruoklių, kaiščių ir t.t.). - Oro kanalai turi turėti atbulinį ir uždarantįjį vožtuvus tokios pat sistemos, kaip ir užpylimo angos ir turėti tokį pat skersmenį. Užpylimo metu ventiliacijos išėjimo angos atitinkamu sujungimu turi būti sujungtos su pagrindiniu rezervuaru arba su skaidriu nešiojamu bakeliu, kurio talpa ne mažesnė nei 20 litrų, tokiu būdu užtikrinant visišką sistemos sandarumą. Užpylimo operacijos pradžioje garų sugaudymo rezervuarai turi būti tušti. Tais atvejais, kai trasoje nėra galimybių dalyviams garantuoti centralizuotos sistemos, jie turi užsipildyti kuro pagal pateiktą procedūrą. Rezervinio bako lygis jokiais atvejais negali būti daugiau nei 3 metrai virš kelio, kuriame vyksta užpylimas. Tas galioja visą lenktynių laiką. Perpildymo bakai turi atitikti vieną iš brėžinių: 252-1 arba 252-2. . | - There must be a gap of at least 20 cm between the right and left mud flaps in front of the rear wheels. - The bottom of these mud flaps must be no more than 10 cm from the ground when the car is stopped, with nobody on board. - Above and over the entire height of the tyre, at least 3/4 of the width of the tyre must be covered (seen from behind). - In vertical projection, these mud flaps must not protrude beyond the bodywork. Mud flaps to prevent splashing towards the front, made from flexible material, may be installed at the front of the vehicle, if the supplementary regulations of the event authorise them or impose them. They must not protrude beyond the overall width of the vehicle, or beyond the original overall length by more than 10 cm, and at least one third of the width of the car must be free in front of the front wheels. ARTICLE 8 : ELECTRICAL SYSTEM 8.1 Lighting A fog light may be changed for another light, and vice versa, provided that the original mounting remains the same. 8.2 Alternators and Alternator-starters The mounting of the alternators and alternator-starters are free. 8.3 Horn In rallies only, the noise level produced by the horn must be greater than or equal to 97 dB during at least 3 seconds, measured 7m in front of the vehicle. ARTICLE 9 : FUEL - COMBUSTIVE 9.1 The fuel must be commercial petrol which comes from a service station pump, without any additive other than that of a lubricant on current sale. The fuel must comply with the following specifications: - 102.0 RON and 90.0 MON maximum, 95.0 RON and 85.0 MON minimum for unleaded fuel. - 100.0 RON and 92.0 MON maximum, 97.0 RON and 86.0 MON minimum for leaded fuel. The measurements will be made according to the standards ASTM D 2699-86 and D 2700-86. - Density between 720 and 785 kg/m3 at 15°C (measured according to ASTM D 4052). - A maximum of 2.8 % oxygen (or 3.7 % if the lead content is less than 0.013 g/l) and 0.5 % nitrogen by weight, the remainder of the fuel consisting exclusively of hydrocarbons and not containing any power-boosting additives. The measuring of the nitrogen content will be carried out according to the standard ASTM D 3228 and that of the oxygen content by elemental analysis with a tolerance of 0.2 %. - Maximum content of peroxides and nitrooxide compounds: 100 ppm (ASTM D 3703 or in the case of impossibility UOP 33-82). - Maximum lead content: 0.40 g/l or the standard of the country of the event if it is lower (ASTM D 3341 or D 3237). - Maximum benzene content: 5 % in volume (ASTM D 3606). - Maximum Reid vapour pressure: 900 hPa (ASTM D 323). - Distillation at 70°C: 10 % to 47 % (ASTM D 86). - Distillation at 100°C: 30 % to 70 % (ASTM D 86). - Distillation at 180° C: 85 % minimum (ASTM D 86). - Maximum final boiling point: 225°C (ASTM D 86). - Maximum residue: 2 % volume (ASTM D 86). The fuel being accepted or rejected according to the standard ASTM D 3244 with a confidence limit of 95 %. For vehicles with a catalytic converter, leaded petrol is forbidden. If the fuel available locally for the event is not of a sufficient quality for use by competitors, the ASN of the organising country must ask the FIA for a waiver in order to enable the use of fuel not corresponding to the characteristics defined above. 9.2 Diesel For Diesel engines, the fuel must be gas oil corresponding to the following specifications: - Hydrocarbon level, % by weight 90.0 min. - Density, kg/m3 860 max. - Cetane number (ASTM D 613) 55 max. - Calculated cetane number 55 max. (ASTM D 976-80) - Sulphur content 50 mg/kg max. (pr-EN-ISO/DIS 14596), according to directive 98/70/CE 9.3 Oxydant Only air may be mixed with the fuel as an oxidant. 9.4 Refuelling procedure Standardised coupling : - In case of a centralised system provided by the circuit or a system provided by the competitors, the refuelling hose shall be provided with a leak-proof coupling to fit the standardised filler mounted on the car (in accordance with Drawing 252-5; the interior diameter D must not exceed 50 mm). - All cars must be provided with a fuel filler complying with this diagram. This leak-proof fitting must comply with the dead man principle and must not therefore incorporate any retaining device when in an open position (spring-loaded, bayonet, etc.). - The air vent(s) must be equipped with non return and closing valves having the same closing system as that of the standard filler and having the same diameter. During refuelling the outlets of the air-vents must be connected with the appropriate coupling either to the main supply-tank or to a transparent portable container with a minimum capacity of 20 litres provided with a closing system rendering it completely leak-proof. The venting catch tanks must be empty at the beginning of the refuelling operation. In the cases where the circuits are unable to provide the entrants with a centralised system, they will have to refuel according to the above procedure. The level of the reserve tank may in no case be more than 3 metres above the level of the track where the refuelling is effected. This applies to the whole duration of the event. The overflow bottles must conform to one of the Drawings 252-1 or 252-2. |
| 252-1 | 252-2 |
| Rezervinis bakas ir visos metalinės užpylimo sistemos dalys nuo sujungimo , per skaitiklį iki rezervuaro ir jo stovo turi būti įžeminti. Rekomenduojama: 1. Kiekvienas degalų punktas turi turėti dvi aviacinio tipo įžeminimo jungtis. 2. Užpylimo sistema (įskaitant kolonėlę, rezervuarą, žarnas, antgalius, vožtuvus ir sugaudymo bakus) turi būti prijungta prie vienos aukščiau minėtos įžeminimo jungties. 3. Automobilis, stovėdamas kuro užpylimo punkte, turi būti prijungtas prie kitos įžeminimo jungties. 4. Užpylimo žarnos neturi būti prijungiamos tol, kol neįvykdytos 2 ir 3 sąlygos. 5. Visi užpylimo punkto komandos nariai turi dėvėti antistatinius apsauginius drabužius. Užpylimo bakas turi būti vienas iš tokių: -modelis pagamintas iš gumos, FT3 , FT3 1999 arba FT5 tipo, pagamintas aprobuoto gamintojo; arba -bakas, atitinkantis vieną iš brėžinių: 252-3 arba 252-4. | The reserve tank and all metal parts of the refuelling system from the coupling over the flow meter up to the tank and its rack must be connected electrically to the earth. The application of the following is recommended: 1. Each pit should be equipped with two aircraft type grounding connections. 2. The refuelling system (including tower, tank, hose, nozzle, valves and vent bottle) should be connected to one of the above grounding connections for the entire duration of the race. 3. The car should be connected, at least momentarily, to the other grounding connection as soon as it stops in the pit. 4. No fuel hose connection (fill or vent) unless and until conditions 2 and 3 have been fulfilled. 5. All fuel-handling pit crew members should wear non-static protective clothing. The refuelling tank may be one of the following: - models made of rubber, of the type FT3 1999, FT3.5 or FT5, built by an approved manufacturer, or - tanks conforming to one of the Drawings 252-3 or 252-4. |
| 252-3 | 252-4 |
| Pritaikymas: Turistiniai automobiliai (Grupė A), Didžiojo turizmo automobiliai (Grupė B), žr. bendruosius reikalavimus FIA čempionatams. 9.5 Bako ventiliacija Leidžiama įrengti bako ventiliaciją, išeinančią per automobilio stogą. 9.6 - FT31999, FT3.5 ar FT5 bako įrengimas FT31999, FT3.5 ar FT5 bakas gali būti įstatytas originalioje vietoje arba bagažinėje. Turi būti anga, skirta pašalinti į bako skyrių patekusį kurą. Užpylimo angos padėtis ir matmenys, o taip pat jos dangtelis gali būti pakeisti, jei tik nauja instaliacija neišsikiša iš kėbulo ir garantuoja, kad kuras netekės nei į vieną iš vidinių automobilio skyrių. Jei užpylimo anga yra automobilio viduje, ji turi būti atskirta nuo kabinos skysčio nepraleidžiančia pertvara. 10) STRAIPSNIS : Stabdžiai Anglies pluošto stabdžių diskai yra draudžiami. 11) STRAIPSNIS : Energijos kaupimas Bendras rekuperuojamos energijos kiekis, kaupiamas automobilyje, negali viršyti 200 kJ; ši energija gali būti vėl panaudojama, tačiau neviršijant 10 kJ energijos, reikalingos 1 kW galios išgavimui. 12) STRAIPSNIS : Vėdinimas Kitu tikslu, negu vairuotojo atvėsinimas, neleidžiama automobilio viduje arba išorėje vežtis vėsinimo medžiagų nei kietu, nei skystu, nei dujiniu pavidalu. | Application: For Touring Cars (Group A), Grand Touring Cars (Group B), refer to the general prescriptions of the FIA Championships. 9.5 Tank ventilation It is authorised to equip a tank with ventilation exiting through the car roof. 9.6 Installation of the FT3 1999, FT3.5 or FT5 tank The FT3 1999, FT3.5 or FT5 tank may be placed either in the original location of the tank or in the luggage compartment. There must be an orifice to evacuate any fuel which may have spread into the tank compartment. The position and the dimension of the filler hole as well as that of the cap may be changed as long as the new installation does not protrude beyond the bodywork and guarantees that no fuel shall leak into one of the interior compartments of the car. If the filler hole is situated inside the car, it must be separated from the cockpit by a liquid-tight protection. ARTICLE 10 : BRAKES Carbon brake discs are forbidden. ARTICLE 11 : ENERGY STORAGE The total quantity of recuperated energy stocked in the car must not exceed 200 kJ; this energy may be re-used without exceeding 10 kJ by means of 1kW maximum. ARTICLE 12 : COOLING Except for the sole purpose of cooling the driver, the transporting and/or use of any solid, liquid or gas cooling agent, whether inside or outside the car, is prohibited at all times throughout the event. |