Additives and corrections to

FIA Appendix J Article 261

Specific Regulations for Production Cars on Circuits

(Super-Production)

ARTICLE 2: HOMOLOGATION

At least 2500 fully identical units must have been produced in 12 consecutive months and homologated by the FIA in Production Cars (Group N) or ASN’s.

Supply Variants (VF) homologated in Touring Cars (Group A) are also valid in Production Cars (Group N).

All Production Variants (VP) are valid in Super Production Cars (Group SP).

Option Variants (VO) of the Touring Cars (Group A) form, without a minimum production, shall not be valid in Super Production Cars (Group SP), unless they refer to:

- fuel tank;

- safety cage;

- seat supports and anchorages;

- safety harness mounting points;

- 2/4-door versions.

The use of tanks homologated in VO on the Touring Car (Group A) form must be carried out under the conditions of Article 5.9.2 of  the Touring Car (Group A) regulations, and Article 254-6.8.

Option Variants (VO) of the Production Cars (Group N) form, without a minimum production, shall not be valid in Super Production Cars (Group SP).

Evolutions of the type (ET), kit variants (VK) and sporting evolutions (ES) homologated in Group A are not valid in Production Cars (Group N), neither in Super Production.

Nevertheless, evolutions of the type and the sporting evolutions homologated in Group A as from 01.01.97 are valid in Group N.

The basic car must have a cubic capacity between 1700 cm³ and 2300 cm³. If the car is originally equipped with a supercharger system (for example turbo), the system must be removed.

For any car failing to hold an FIA homologation, the manufacturer or the general importer must submit an application for an ASN’s homologation. For this purpose, the car must be described on an ASN’s Homologation Form. The ASN’s homologation regulations are available from the ASN’s secretariat.

In order to be granted an ASN’s homologation, the car must have a General (ABE) or EU Certification and comply with the following requirements.

- The basic car must have two-wheel-drive (rear or front wheel drive).

- Basic cars with engines having more than two throttle valves for the load regulation in their standard version are not admitted. Throttle valves, which modify the length resp. the diameter of the inlet ports in a variable induction pipe, are not considered to be load regulating throttle valves.

ARTICLE 7: ENGINE

7.2 Ignition

The make and type of the spark plugs and ignition coils, rev limiter and leads are free.

The ignition components in the electronic control unit are free.

The engine speed is not limited.

ARTICLE 10: WHEELS AND TYRES

10.1 Complete wheel

The maximum dimensions of the wheels (rims + flanges) are:

a) 7" x 15" and their weight must not be less than 5 kg. The complete wheel inflated to a pressure of 2 bars must fit into a box of a diameter of 580 mm and a thickness of 223 mm.

b) 9" x 17" and their weight must not be less than 9 kg each. The complete wheel inflated to a pressure of 2 bars must fit into a box with a diameter of 610 mm and a thickness of 250 mm.

The diameters measured at the level of the inner and outer rim edges of a wheel must be identical, with a tolerance of +/- 1.5 mm.

In all other respects the wheels are free provided that they are made of metal and that they are made in a single unit.

Wheel fixations by bolts may be freely changed to fixations by pins and nuts provided that the number of fixation points and the diameter of the threaded parts remain unchanged.

The fitting of air extractors on the wheels is prohibited.

The upper part of the complete wheel (flange + rim + tyre), located vertically over the wheel hub centre, must be covered by the bodywork when measured vertically.

Foam or any other system enabling the car to be driven without air in the tyres is prohibited.

All pressure regulations systems are prohibited.

ARTICLE 18: LANGUAGE

The English version of these regulations shall be considered as the authentic text in the event of a dispute.